This is the updated 1995 covenants that include Addendums 1, 2, 3, 4 and 5, including updated Bylaws and Building Design Requirements, all which were approved by previous Sunstone Homeowners Association, which you can find on the DANA Properties website under Sunstone.

EXHIBIT “A” TO THE DECLARATION OF COVENANTS, CONDITIONS,

AND RESTRICTIONS FOR BLOCK 1, SUNSTONE ADDTION

SITE WORK

AND

BUILDING DESIGN REQUIREMENTS

“Committee” shall mean the Architectural Design Committee (ADC) or Architectural Review Committee (ARC) (the ADC and ARC are one and the same) for the subdivision.

The following requirements for the design and construction of Structures within Sunstone are part of the described Declaration and shall be strictly observed by all Owners and/or Builders, their contractors, subcontractors, employees, and agents engaged in the construction of improvements:

Section 1.   	Design Review Procedure. The Building Design Requirements have been established to provide standards to the Lot Owners at Sunstone. Their purpose is to help each Lot Owner discover the unique opportunities of his individual site through the use of design principles used by architects, interior designers, land planners and landscape architects. The Design Committee has been established to administer the Building Design Requirements and to review plans for conformity with those requirements. The purpose of the review is to help each Owner and to bring to the attention of the Lot Owner or Contractor and his Architect the many Sunstone benefits. The Design Committee will not seek to unreasonably restrict the taste or individual preferences; however, they are charged with notifying the Owners Association whenever these standards are not being followed.

- Project Design Guidelines:

Subdivision Design Guidelines. Sunstone is a de minimums Planned Unit Development (PUD) with a maximum of 32 residential home lots developed on approximately half of an 11-acre site. There is no common clubhouse, pool, or active recreation site planned, however this will not prevent the Owners Association from developing such facilities. The majority of the remaining acres will be held in perpetuity in the open space in the lower canyon area of lot #37 will be pathways in the canyon with some type of enhanced or native shade trees and landscaping, one or two rest areas in the canyon with some type of seating (emphasizing an oasis concept), and a simply system of small stepping-stone ponds and dams that will be dry most of the year and will help retain small amounts of water to nourish native landscaping following a storm. All areas of lots 1-18 below the lower retaining walls are intended to remain in desert landscape to blend with the lower canyon are of lot #37, however, some type of enhanced desert plantings may be added if approved by the design committee. On the upper rim above the canyon, between Lot 7 & 8, some type of trees or gazebo or shade structure and some type of seating and landscape may be employed. The upper rim will have a more heavily planted and maintained landscape intermixed with the native desert environment, while still emphasizing an oasis concept. The median, entry gate and perimeter walls will be landscaped and use Santa Barbara style architectural design features complimentary and consistent with the unified design theme of Sunstone.

Home Design Guidelines. The exterior architectural style for Sunstone is generally referred to as “Santa Barbara style,” a traditional Spanish or Mediterranean style common to the central and southern California coastal region. The buildings are characterized by low pitched tile roofs. Some examples feature short eaves (having an overhang of less than two feet), with detailed plaster facial/soffits that look like a cornice, while other examples have long eaves with corbel-cut vigas and tongue-and-groove exposed soffit decking. The stucco colors vary from off-white to pale pastel shades; most have a mottled appearance giving the buildings an aged look. Many from elevations employ such features as large-tile-roof-covered turrets or rotunda entries, wrought iron verandas, half columns protruding walls, carved timbers and corbels, and tall tile-capped chimneys as well as other combinations of Moorish, Spanish, French and Italian design details. (The Developer and Project Architect have examples available for review.)

- Project Architect:

The El Paso architectural from of Wright and Dalbin has been named original “Project Architect,” and the Architectural Design Committee has appointed the Project Architect to act for the Committee as a whole. The Project Architect serves at the pleasure of the Developer or the Board of Directors of the Sunstone Homeowners Association. However, successor Project Architects shall be obligated to continue following the “Santa Barbara” design style.

- Project Landscaper:

A Master Landscape Plan with an approved list of plants has been approved by the Architectural Design Committee. Sally Homan of “Classic Landscapes,” has been selected to be “Project Landscaper” and to design and install all front yard and common area landscaping, so that uniform appearance can be established. All front yard and common area landscaping, as well as private side and rear yard landscaping, will only be installed after approval by the Architectural Design Committee. (This is not meant to prevent each individual homeowner from expressing individual design preferences, but rather to direct the smooth transition in landscaping from lot to lot.) From time to time the Developer or the Board of Directors of the Sunstone Homeowners Association may choose to change the project Landscaper or modify the Master Landscape Plan; however, in no case shall any change in the Project Landscaper or the Master Landscape Plan alleviate the need and requirement for a cohesively designed Project Master Landscape Plan.

- Initial Design Review Process:

- All Preliminary designs shall be prepared by the Project Architect in consultation with the Lot Owner.

- All front yard and common area landscape shall be designed and installed by the Project Landscaper in consultation with the Lot Owner.

- At the Owners/Contractors’ option, Working Drawings may be prepared by the Project Architect or by a third party; however, final approval of the working drawings will be considered on the basis of conformity with the approved preliminary designs developed by the Project Architect.

- All Final Building Plans must be submitted for approval to the Project Architect. All Final Landscape Plans must be submitted for approval to the Project Landscaper. To be approved, both the Architectural and Landscape plans must be substantially the same as the approved preliminary plans; the Project Architect must have approved any building design changes and certify that any modifications or changes thereto are generally in conformance with the Santa Barbara style, and the Project Landscaper must have approved any landscaping design changes and certify that any modifications or changes thereto are generally in conformance with the Master Landscape Plan.

- If there is disagreement between the Lot Owner and either the Project Architect or the Project Landscaper, the Lot Owner shall have the right to appeal to the Architectural Design Committee for a variance.

DESIGN REQUIREMENTS (ARC) have been deleted and replaced with the following:

Section 2.	Appeal and Variance Procedure

The Architectural Design/Review Committee (ADC/ARC are one in the same) serves as the review authority for interpretation of the design requirements. The Board has the responsibility for enforcement of the design requirements. Any property owner requesting a variance must submit same to the ARC for approval, providing a drawing, photograph or sketch as appropriate. If the property owner disagrees with the decision of the ARC, the property owner shall have the right to appeal to the Board for a final decision on the variance.

Section 3.	Required Submissions to the Design Committee

- Submissions of Plans and Specification. Construction of any structures and landscaping on Lots in Sunstone shall be subject to review by the ADC/ARC and submission of the following:

- A Site Plan having a scale of not less than 1” =8’0” showing:

  - The entire Lot including Lot setback and easement lines; and

  - Location of all existing and proposed improvements; and

  - A roof plan of all structures showing location of all plumbing, electrical, solar and mechanical equipment; and

  - Existing grade and all changes in existing grade including contour lines for building sites, driveways and walkways: and

  - Location of all existing and proposed landscaping; and

  - Location of all utilities, including stubs meters, transformers, etc. and the location where utilities meet the structures.

- A floor plan having a scale of not less than 1/8” = 1’0”.

- Exterior elevations at a scale of not less than 1/8” = 1’0” showing locations of all plumbing, electrical. Solar equipment, mechanical equipment, utility meters transformers and trash enclosures.

- Description of all exterior building materials including color samples. (See Attachment “B”, SUNSTONE DEVELOPMENT-COLORS)

- The Lot Owner shall have the burden to show that the design requirements have been met and shall submit a completed copy of Attachment “A”, “Plan & Specification Submittal Checklist”.

- An acceptable schedule of construction shall also be submitted.

- All plans and specifications shall be prepared by qualified persons in a professional manner.

- Final approval of plans will not be granted by the Design Committee until final working plans stamped approved by the City of El Paso and a valid building permit have been submitted to the Design Committee and a determination made that the approved plans are substantially the same as the final plans submitted to the Design Committee.

Section 4.	Design Standards All submissions under Section 3 shall comply with the following:

- Site Details Site details such as transformer enclosure, mailboxes, planters, driveways, sidewalks, swimming pools, spas, etc. shall be given special architectural and landscaping design consideration to either screen them from view of improve their aesthetic appeal.

Section B. Rock Walls have been amended to read with the following:

- Rock Walls All rock wall materials used by anyone for the Sunstone Subdivision that may be seen from any street, any lot within the subdivision or any common area in the subdivision must generally be in shades of gray and match the color and texture of the rock used by the developer in the entry gate. In no case shall the rock to be used on any lot that would be exposed to the public view contain shades of red, yellow, green, white, etc. All rock walls will be constructed with a concealed mortar joint in a Dry-Stack manner. The only exception to the concealed mortar joint construction requirement shall be for retaining walls where it is determined that for structural reasons the use of a solid pointed wall would be more appropriate or if an existing rock wall requires repair due to failure.

Section C. Street Lighting and Exterior Bracket Type Lighting has been amended to read with the following:

- Street Lighting and Exterior Bracket Type Lighting. The standard exterior light fixture used initially in the subdivision is no longer available. If a property owner has the need to replace their streetlights, a request shall be submitted to the ARC for review and approval. In these cases, all exterior light fixtures at that property must be replaced. Each home shall have at least one of these fixtures, connected to a photo-electric cell fitted with a 9-watts compact fluorescent bulb, or the equivalent LED light, installed at a maximum distance of 20’0” from Sun Point Lane.

Section D. Address Numbers has been amended to read with the following:

- Address Numbers Every home within Sunstone will use 4-to-5-inch address numbers made of ceramic or brass that complement the existing colors and design of the house exterior.

- Maximum Site Coverage No more than 75% of each Lot shall be covered with impermeable materials.

- All utilities located on all lots must be installed underground and must meet the requirements of the city of El Paso.

- Sight Lines, Visibility, Shrubs, and Trees No shrub or tree planting which obstructs sight lines at elevation between tow and six feet above the roadway shall be planted or permitted to remain on any corner Lot within the triangular area formed by the curb  lines of such intersecting streets and a line connecting such curb line at points twenty-five feet from their intersection, or in the case of a rounded corner from the intersection of the curb lines as extended.  The same sight line limitations shall apply on any Lot within ten feet of the intersection of a street curb line and the edge of a driveway.  No trees shall be permitted to remain within such distances of such intersections unless the foliage line in maintained at a height of more than six feet above ground level.

Section H. Fences, Hedges, Privacy Garden and Retaining Walls has been amended to read with the following:

- Fences, Hedges, Privacy Garden and Retaining Walls All property divisions, fences, garden walls, retaining walls, etc. shall be made of stone or rock masonry, CMU covered with stucco, or for the rear of lots 1-18 and all lot areas adjoining the common area (Lot #37), a combination of stone masonry and wrought iron (see sheet 6 or 10, Sunstone Addition Replat “A” Subdivision Improvement Plan).  All stone shall generally be in natural earth-desert tones, such as shades of gray, brown, or a combination of the two.  All Stucco walls must match the color and texture of the residence or which they are a part (See Attachment “B”, Sunstone Development Colors).  The wrought iron used for lots 1-18 and adjacent to the common areas Lot #37 must be in the same color, style, size, and general dimension for all lots as approved by the Design Committee (See Attachment “B”, Sunstone Development Colors).  No wire or chain link fences shall be permitted on any lot except for special situations such as sport courts, dog runs, etc., when approved by the Variances of the Design Committee.  Wire or chain link fences shall be used only if each section of fence is strung between columns or masonry; no pipe or metal columns shall be allowed.  The fence material itself must be either covered with a vinyl or plastic coating in approved Sunstone Development Colors.  Exposed galvanized metal products shall not be used for fences or gates.

- Landscape

Section Lush Green Landscaping has been amended to read with the following:

(1) Lush Green Landscaping: All front yards and rear yards for lots 1-32 and upper-level Common Areas shall be landscaped in a lush green manner employing the use of lower water-use plants.  For the purpose of establishing a uniform street appearance, all front yards shall be designed by the Project Landscaper before plans are submitted to the Design Review Committee and installed by the Project Landscaper before any home if occupied.  The side and rear yards shall be designed and submitted to the Architectural Design Committee within 30 days of occupancy and installed within 180 days after approval.  The appearance of the rear yads of Lots 1-18 as viewed from surrounding streets will be of particular importance to the Design Review Committee so that continuity and uniformity of appearance can be maintained.  Any change to the front-side yard landscaping or architecture, if visible from the street, other than exact plant replacement, must be submitted to the ARC for review and approval.

(2) Desert Landscaping may be used on Lots 1-32 and within the upper level secured common area under the following conditions:

a.  Desert Landscaping shall be used only for accents, perimeter landscape along Irondale Street and in canyon areas.

b.  Desert Landscaping used for accents on Lots 1-32 shall not exceed more than 20% of the air-conditioned, ground floor area of the Residence.

c.  Each Lot Owner shall be responsible for flooding which may be caused by impermeable materials used on the Lot and the raising of the Lot grade.

(3) Prohibited Plants: Use of the following is prohibited and shall not be planted or allowed to grow on any Lot:

a.  Trees

Salt cedar

Cottonwoods, native female

Mullberry, fruiting

Poplars, Carolina Balm, O’Gilead, and Lombardy

Elms

b.  Plants

Bamboo: Invasive/Running (Non-invasive or clumping Bamboo is permitted)

Ivy (or similar evergreen climbing plants)

(Note: No new Ivy shall be planted, with one exception: Ivy may be planted for erosion control at the east end embankment across from #14 and #16.  Ivy on the Irondale wall was left at the request of the respective homeowners.  The rest of Irondale wall Ivy was removed, and wall repairs made by the Sunstone Homeowner’s Association as a one-time effort.  All future Irondale wall repairs are the responsibility of the respective property owners.  If a homeowner has or wants vines, they are not allowed in any area that the Sunstone Homeowner’s Association is responsible for.

- Steet Access. Lots shall have access only from Sun Point Lane.

- Grading, Storm Water, and Irrigation. Each Lot shall be designed to the following criteria:

(1) Positive grades shall move water away from each home, garage, walk, and driveway to small, shallow depressions on the Lot.

(2) Each lot shall be required to have two shallow depressions, one in the front yard and one in the rea or side yard, designed to retain normal irrigation water run-off and moderate amounts of rainfall before any excess water runs onto the street.

(3) Each Lot Owner and contractor is encouraged to use culverts under drives and walks to keep water from ponding on or flowing across walk and drives.

(4) Each Lot Owner shall be responsible for any water damage to other Lots caused by water moving from his Lot (See ATTACHMENT “C”, Minimum Recommended Retaining Wall Moisture Proofing and Drainage Cross-Section).

L. Setback Lines No Structure shall be erected or placed within the setback areas except that detached garages, storerooms, guest houses, and other auxiliary buildings may be located within setback areas if approved by the Design Committee.  The following setback lines on the Property are hereby established:

(1) Front Yard Set Back:

a) All garages shall have a minimum driveway length or 20’-0” as measured along the center line or a 2-car driveway.

b) The Structure must be set back not less than 10’-0” from the street for single-store portions of any building and not less than 12’-0” for any two-story portions of any building.

(2) Side & Rear Yard Set Back: A ten (10) foot minimum rear yard and side yard set back shall be required for Residences except where Zoning Ordinance of the City of El Paso may require a greater setback for lots adjacent to neighboring properties outside of the subdivision, not zoned the same as Sunstone.

M.	Minimum Square Footage Each home shall have a minimum air-conditioned area of 1650 square feet.

N.	Garage/Driveway Size and Design Each home shall have a 2-car garage and shall have a wide and long enough driveway to allow 2 cars to park side by side off the street.  The garage may be attached or freestanding and shall have a minimum area of 400 square feet including outside walls.  Such a garage shall be constructed with a 20-foot minimum width and be designed to allow 2 cars to park side by side, moving in and out of the garage without obstruction.

O.	Height Limitation No structure may be built having a height more than 30 feet above the curb elevation for the Lot.

P. 	Utility Meter Enclosures Utility meters shall be enclosed in a closet or screened from view in a manner architecturally compatible with the main structure.

Q.	Trash Receptacles and Collections Trash, garbage, and other waste shall be kept in sanitary containers.  Trash containers shall be enclosed in a closet or other architecturally compatible enclosure, if provided.  Each Owner shall make or cause to be made appropriate arrangements with The City of El Paso for collection and removal of garbage and trash on a regular basis.  Each Lot Owner shall observe and comply with all reasonable regulations or requirements promulgated by The City of El Paso or the Association, or both in connection with storage and removal of garbage.

R.	Roofing

1.  Roofscape Each multi-story Structure shall be required to have a minimum of 65% of the roof area over the air-conditioned space covered with tile roofing materials and have a minimum pitch of 3.5/12, excluding second floor balconies and terraces.  Each single-story Structure shall be required to have a minimum of 75% of the toral roof area covered with tile roofing materials and have a minimum pitch of 3.5/12.

2.	Roof Pitch All roofs shall have pitches of either 6” in 10’0” or less, or greater than 3’6” in 12’0” (3.5/12).

3.  Approved Roofing Material for Semi-Flat Roofs Any roof with a pitch of 5’ in 10’0” or less may be covered with one of the following materials: formed modified asphalt sheathing, rolled roofing, neoprene or other synthetic rubber watertight roofing, gravel surfaced built up roofing, or any Class “A” approved watertight roofing material.  The color of the material must be approved by the Design Committee.

4.	Approved Roofing Material for Pitched Roofs: Any roof with a pitch of 3’-6” in 12’-0” (3.5/12) or greater may be covered with one of the following materials: glazed cement or clay slate roofing, glazed cement of clay tile roofing. Copper or zinc roofing trim may be used provided approved by the Architectural Design Committee. Other materials may be used upon approval of the Design Committee only after issuance of a written Variance. All tile roof materials & colors are listed on Attachment “B” SUNSTONE DEVELOOPMENT – COLORS and the material shall be installed in a good and workmanlike manner. No gravel, crushed stone, galvanized or rusting materials, asphalt rolled roofing or standard weight asphalt shingle shall be allowed on pitched roofs. No roofing product shall be applied without written approval of the Design Committee or its agent.

(It is the desire of the Developer to have the Project Architect select 2 or 3 acceptable tile roof colors that are manufactured by 2 or 3 different sources for this project and to have the same 2 or 3 tile roof colors arranged to complement 3 or 4 exterior color designs for the entire Sunstone Project. See SUNSTONE DEVELOPMENT COLORS, ATTACHMENT “B”)

S.	Mechanical Equipment, Vents, Skylights, Solar Equipment, Chimneys, Etc.

Section 1. General has been amended to read with the following:

- General. No mechanical equipment shall be visible from the street, or any lot, or common area.  Any property owner wishing to install solar equipment must submit a written application to the ARC for review and approval.  No solar equipment shall be installed without the prior written approval of the ARC.

- Mechanical Equipment Evaporative coolers, refrigeration units, heat pumps, etc. shall not be permitted on pitched roofs.  They shall be permitted on semi-flat roofs only if no other reasonable alternative exists and shall be shielded from view from streets and adjacent lots by enclosures made of the same roofing and wall materials identical in finishes as the exterior walls and roof of the structure to which they are attached.  All mechanical equipment, regardless of location, shall be screened from view.  Exposed mechanical equipment will not be approved.

- Vent Stacks & Flues Mechanical and plumbing vents and flues shall be collected into a minimum number of chimney-like chases and placed on the roof where their visual impact is minimized.  The chases shall be made of the same materials as the roof and walls of the home.  Exposed mechanical & plumbing vents and flues, etc. on pitched roofs will not be approved.  Exposed mechanical & plumbing, vents, and flues may be screened behind parapet walls on semi-flat roofs.  All exposed metal parts must be painted to match the exterior walls.

- All metal chimney parts, spark arrestor caps, flue liners, etc. shall be painted and screened to be hidden from view.  Each builder is encouraged to trim the top of each chimney with roof tile matching the roof of the home to which the chimney is attached.  Paint will not, in and of itself, classify as screening.  Exposed metal chimney parts will not be approved.  (See Attachment “D” Recommended Conceptual Chimney Screening Details)

- Skylights and Solar Collectors Skylights and solar collectors shall be permitted only if they are located on the rear of the home, out of side from the streets, structures, adjoining lots, and common areas.  The Design Committee recommends the use of clear story windows and dormers as an alternative to skylights.

Section T. Color Coordination has been amended to read with the following:

T.	Color Coordination All exterior finish materials shall be color coordinated with each individual Structure and as to the effect on other structures in the neighborhood.  An approved color selection list has been made by the Project Architect.  All exterior colors and finishes shall be approved by the Architectural Design Committee (See ATTACHMENT “B” SUNSTONE DEVELOPMENT COLORS).  All fences visible from the front of the homeowners’ property, that were not in compliance with the previously approved colors (or painted in several color tones), shall be repainted, and brought into compliance with the new colors specified in Attachment B.

U.	Exterior Wall Materials. Exterior wall materials of any Structure may be of stucco or thin coat plaster veneer, with trims in stone precast concrete, brink, slump or mission stone, or stabilized and sunbaked adobe, or wood or wood siding.  Other materials may be used only upon the issuance of a Variance from the Design Committee.

V.	Exterior Trim Materials The exterior trim material (facia, soffit, etc.) shall be of low maintenance or maintenance-free materials.  The Design Committee urges the use of fibered cement board, treated wood, redwood, cedar, aluminum and plastics to reduce the maintenance on exterior trim, or the use of plaster facia/soffit that are integrated into decorative cornice designs.

W.	Miscellaneous Provisions

1.  Deed Restrictions & Easements:

(See Covenants, Conditions, & Restrictions)

2.  Trucks, Buses, R.V.s, Boats, Motorcycles, etc.:

(See Covenants, Conditions, & Restrictions)

ATTACHMENTS:

- ATTACHMENT “A”: Plan & Specification Submittal Checklist

(7 Sheets)

EXHIBIT “A” to the Declaration of Covenants, Conditions, and Restrictions for Block 1, Sunstone Addition Site Work and Building Design Requirements is hereby amended by the addition of the following to Attachment “B”:

- ATTACHMENT "B”: SUNSTONE DEVELOPMENT – COLORS

(Sheet 1 of 1)

Note: The colors listed in Attachment B are outdated and are no longer available.

If a property owner wishes to repaint or re-roof their house, paints or roofing material that match the existing color/texture/design shall be used. If a different color/texture/design is desired, a variance request must be submitted to the ARC for review and approval.

The following fence/gate colors or color matched equivalents are the only ones approved for painting of the fences and/or gates:

Green Color selected is Sherwin Williams SW7742 Agate Green

Tan color selected is Sherwin Williams SW9103 Farro

Sherwin Williams Sunstone Green

- ATTACHMENT “C”: Minimum Recommended Retaining Wall Moisture Proofing & Drainage Cross Section (Sheet 1 of 1)

- ATTACHMENT “D”: Recommended Conceptual Chimney Screening Details

(Sheet 1 of 1)